Highlight

1. differences between effects of 2 layer and 3 layer network and the reasons. One layer can not solve the softening process, since the order is not enough in the softening process.

2. influence of the different training epoch on the smoothness of the top forces curve. If the training epoch is a very large number, then the Newton-Raphson iteration will not converge.

3.  influence of the relative error (1e-2, 5e-2) on the accuracy and the iteration number.

introduction

In this work, we aim to propose the data-driven method as the surrogate model that directly represents the constitutive relationship from the raw data collected from the FEM and DEM coupling multiscale computation random-strain-controlled meta-RVE loading. This model should be capable of capturing the history dependency in data and can be implemented into a FEM and DEM multiscale computer program with minimal intrusion. The memory effect was considered by the internal variable-based history parameterization. The computation method was implemented based on the FEM and DEM coupling platform proposed by Guo [1]. In this work, TENSORFLOW was engaged to construct the neural network to represent the constitutive relationship. The consistent matrix is necessary in multiscale FEM-DEM scheme, where the implicit Newton-Raphson iteration was utilized. This was achieved by the approximate elastic-plastic consistent matrix calculated through the fabric variables in the particle assembly.

Formulation and solution

In the FEM-DEM simulation, a DEM assembly is attached to each Gaussian point in the finite elements which serves as the representative volume element (RVE) at the specific point to capture the local material response. The boundary condition (deformation) for each DEM packing is interpolated from the FEM solution (displacement). Then the stress and consistent matrix are derived and upscaled to the FEM solver to update the solution. The homogenized Cauchy stress of the particle assembly is derived through the Love’s formula [2].

(1)

Where  is the total volume of the assembly,  is the number of contacts within the volume and  and are the contact force vector and the branch vector connecting the centers of the two contacted particles, respectively. The simulations in this paper will be carried out in two dimensional (2D) where the stress and the strain components in the out-of-plane direction are assumed to be zero. Subsequently, two commonly referred quantities, the mean effective stress and the deviatoric stress, are calculated as  and , where  is the deviatoric stress tensor and  is the Kronecker delta.

(2)

Where  and  are the equivalent normal and tangential stiffness modulus and  and  are the unit vectors in the outward normal and the tangential directions of a contact, respectively. The stiffness can be derived as follows:  and  where  and  are the shear modulus and Poisson’s ratio of the particle material, respectively, and  is the common radius of the contacted particles (i.e.  where  and  are the radii of the two contacted particles).

reference

1. 	Guo N, Zhao J (2014) A coupled FEM/DEM approach for hierarchical multiscale modelling of granular media. Int J Numer Methods Eng 789–818. https://doi.org/10.1002/nme

2. 	Christoffersen J, Mehrabadi MM, Nemat-Nasser S (1981) A micromechanical description of granular material behavior. J Appl Mech Trans ASME 48:339–344. https://doi.org/10.1115/1.3157619